ENGINEERING SURVEYING

Introduction to Engineering Survey — Exam Notes

National University of Sciences and Technology (NUST) | Engr. Ameer Hamza

1. DEFINITION OF SURVEY

What is Survey?

Survey is the science of measurement with the objective of increasing our knowledge about the shape of the earth and features upon it.

Two Main Categories:

Plane Survey: Survey of small areas where the earth's surface is assumed to be flat. Earth's curvature is ignored.

Geodetic Survey: Survey of large areas where the earth's curvature cannot be ignored and must be accounted for.

📝 Note: A person of 2m height can see only up to 5.04 km before the earth's curvature cuts off their line of sight. Formula: X = sqrt[(R+H)^2 - R^2] where R = 6372797 m.

2. TYPES OF SURVEY

1. Geodetic Survey

- Determines the size and shape of the earth.

- Provides the main control framework within which all other surveys (topographic, cadastral, engineering) are located.

2. Topographical Survey

- Measures natural and artificial features of the earth's surface.

- Produces maps showing these features.

3. Cadastral Survey

- Defines, demarcates, measures and records the boundaries of properties.

- Overlaps with topographical survey when boundaries follow physical features.

- Done at larger scales than topographical mapping.

4. Engineering Survey

- Conducted to supply specific information for engineering projects (e.g., roads, bridges, dams).

- Usually at larger scales than topographic maps.

- Sometimes requires high accuracy — e.g., measuring a gap for a bridge.

5. Topographic Map

- Shows all information about natural and artificial objects.

- Scale ranges from 1:5,000 to 1:1,000,000 depending on purpose.

| Survey Type | Scale | Purpose |
| --- | --- | --- |
| Geodetic | Very small | Shape & size of earth, main framework |
| Topographical | 1:5,000 – 1:1,000,000 | Maps of natural & man-made features |
| Cadastral | Larger than topo | Property boundary demarcation |
| Engineering | Larger than topo | Specific info for engineering projects |

3. PRINCIPLES OF SURVEY

1. Control

A framework encompassing the entire area must be established first. It should be error-free and all subsequent work is adjusted to it. This is the concept of 'working from whole to part'.

Methods of Control:

a. Triangulation

- One baseline (accurately measured) is used.

- All other distances and positions are calculated by measuring angles from the baseline using triangles.

- Used in: Great Trigonometric Survey of India, completed in 1882 by General J.T. Walker — provided control points across the subcontinent.

b. Traverse

- A connected series of lines (traverse legs) on the earth's surface.

- Both length and bearing of each line are measured.

- Starts from a baseline and ends at a finishing line.

- Points along the route are called traverse stations.

c. Trilateration

- Locates control points by measuring distances only (not angles).

- Uses the geometry of triangles.

- Contrast with triangulation: triangulation measures angles; trilateration measures distances.

d. Astronomical Determinations

- Determines position of a control point in geographic coordinates (latitude and longitude).

- Uses astronomical measurements (e.g., observing star positions like Polaris).

e. Satellite Triangulation (GPS/GNSS)

- Uses signals from multiple satellites to determine precise ground locations.

- Overlapping satellite coverage defines a point's position on earth.

| Method | What is Measured? | Key Feature |
| --- | --- | --- |
| Triangulation | Angles | One baseline + all angles |
| Traverse | Lengths + Bearings | Series of connected lines |
| Trilateration | Distances only | No angles measured |
| Astronomical | Star/celestial observations | Gives lat/long coordinates |
| Satellite | Satellite signals | Modern GPS-based positioning |

2. Economy of Accuracy

- Higher accuracy requires more time, more cost, and more refined instruments.

- Use only the level of accuracy that the project actually requires.

3. Consistency

- The same standard of accuracy must be maintained throughout the entire project.

- Mixing different accuracy levels in one project leads to errors.

4. The Independent Check

- Always use self-checking methods to catch errors.

- Measure distances in both (forward and reverse) directions.

- Use different units of measurement to cross-verify.

- Have computations done by another person using a different method.

5. Revision

- Always conduct the survey keeping future revision and updating in mind.

6. Safe Guarding

- Field notes should be easy for another person to understand.

- Use permanent marks as control points.

- Keep duplicate (hard) copies of field records.

- Maintain soft (digital) copies of all records for backup.

4. QUICK REVISION — KEY FACTS

| Topic | Key Point to Remember |
| --- | --- |
| Survey definition | Science of measurement of earth's shape and features |
| Plane survey | Small area, earth assumed flat |
| Geodetic survey | Large area, curvature accounted for |
| Visibility limit | A 2m tall observer can see ~5.04 km |
| Triangulation | Measures angles from a baseline |
| Trilateration | Measures distances, NOT angles |
| Traverse | Connected lines with known length and bearing |
| Great Trig Survey | Completed 1882 by Gen. J.T. Walker |
| Working principle | Whole to part (control first, detail later) |
| Topo map scale | 1:5,000 to 1:1,000,000 |

5. POSSIBLE EXAM QUESTIONS

- Define survey. Differentiate between plane survey and geodetic survey.

- List and explain the types of survey with examples.

- What are the principles of survey? Explain each briefly.

- Differentiate between triangulation, traverse, and trilateration.

- What is meant by 'working from whole to part'? Why is it important?

- A surveyor stands 2m tall. How far can they see before earth's curvature blocks their view? (R = 6372797 m)

- What is the Great Trigonometric Survey? Who completed it and when?

- Why is an independent check important in surveying?

End of Notes — Good Luck!