LEVELLING

Surveying (CE-183) — Complete Exam Notes

Topics Covered 1. What is Levelling? 2. Key Terms & Definitions 3. Equipment Used 4. Bench Marks (4 Types) 5. Steps in Levelling 6. Simple & Differential Levelling 7. Reduction Methods (HI & Rise/Fall) 8. Adjustments & Error Checking 9. Sources of Errors

# 1. WHAT IS LEVELLING?

Definition: Levelling is the art of finding the relative heights (elevations) of points on the earth's surface. It deals with measurements in a vertical plane.

Simple way to remember:

- We want to know: 'Is point A higher or lower than point B, and by how much?'

- All measurements are vertical (up/down), not horizontal (left/right)

# 2. KEY TERMS & DEFINITIONS

Learn these terms — they are frequently asked in exams!

| Level Surface | Any surface parallel to the earth's curved shape. Every point on it is equidistant from Earth's centre. (Like the surface of a calm lake) |
| --- | --- |

| Level Line | A line lying in a level surface — it curves with the earth and is always perpendicular (90°) to the plumb line |
| --- | --- |

| Horizontal Line | A straight line tangent to the level surface at one point. It does NOT curve — it goes straight outward |
| --- | --- |

| Datum | An assumed reference surface from which all elevations are measured. Usually Mean Sea Level (MSL) |
| --- | --- |

| Elevation / R.L. | The vertical distance of a point ABOVE or BELOW the datum. Also called Reduced Level (R.L.). Positive = above datum, Negative = below datum |
| --- | --- |

| Bench Mark (B.M.) | A fixed reference point of KNOWN elevation. Used as starting/ending point in levelling surveys |
| --- | --- |

| Line of Collimation | The line of sight through the telescope crosshairs. Also called the 'line of sight'. Must be horizontal for accurate readings |
| --- | --- |

| Height of Instrument (H.I.) | The elevation of the plane of collimation (where the instrument sees). Formula: H.I. = R.L. of known point + Backsight reading |
| --- | --- |

| Backsight (B.S.) | Staff reading taken on a point of KNOWN elevation (e.g., a Bench Mark). Also called 'Plus Sight'. It is the FIRST reading after setting up the instrument |
| --- | --- |

| Foresight (F.S.) | Staff reading on a point whose elevation we WANT TO FIND. Also called 'Minus Sight'. It is the LAST reading before moving the instrument |
| --- | --- |

| Intermediate Sight (I.S.) | Any staff reading taken BETWEEN the backsight and foresight from the same instrument position. Used to find elevations of extra points |
| --- | --- |

| Change Point (C.P.) | A point where BOTH a foresight AND a backsight are taken. Used when moving the instrument to a new position. Also called Turning Point (T.P.) |
| --- | --- |

| Station | The point where the STAFF is held (NOT where the instrument stands) |
| --- | --- |

MEMORY TIP — Sight Types: BS (Back Sight) = Known point → Used to calculate H.I. FS (Fore Sight) = Unknown point → Used to find R.L. IS (Inter Sight) = Extra unknown points in between CP (Change Point) = Has BOTH BS and FS — appears TWICE in the table

# 3. EQUIPMENT USED IN LEVELLING

| Equipment | Purpose | Notes |
| --- | --- | --- |
| Engineering Level | Provides horizontal line of sight | Has: telescope, level tube, levelling screws, tripod |
| Levelling Staff | Measures vertical distance | Types: Folding (4m), Invar (3m), Target, Bar Coded |
| Chain / Tape | Measures horizontal distances | Used alongside level readings |
| Level Field Book | Records all readings and notes | Contains BS, IS, FS, HI, RL columns |

## Parts of the Level Instrument

- Telescope — provides line of sight

- Level Tube — makes line of sight horizontal

- Levelling Screws (Footscrews) — centre the bubble

- Tripod — supports the instrument

## Types of Levels

| Wye (Y) Level | Older type; telescope sits in Y-shaped supports; can be rotated |
| --- | --- |
| Tilting Level | Can tilt slightly; used for precise work; has tilting screw |
| Automatic Level | Most common; self-levelling compensator — quick and easy to use |
| Electronic/Digital Level | Reads bar-coded staff automatically; most accurate; stores data digitally |

## Levelling Staff Types

- Folding Staff (4m) — ordinary accuracy work

- Invar Staff (3m) — high precision; does not fold

- Target Staff — observer calls when staff is at correct reading

- Bar Coded Staff — used with digital level; auto-reads electronically

HOLDING THE STAFF CORRECTLY: • Staff must be held PERFECTLY VERTICAL for accurate readings • If tilted, reading will be TOO LARGE (error) • Precise work: use circular level or plumb bob attached to staff • Ordinary work: wave staff forward & backward — take the LOWEST reading (that's when it's vertical)

# 4. BENCH MARKS (B.M.)

A Bench Mark is a permanently fixed reference point of KNOWN elevation above the assumed datum.

| Type | Who Sets It | Key Facts |
| --- | --- | --- |
| G.T.S. B.M. | Survey of Pakistan | Highest accuracy; interval ~100 km across country; elevations refer to Mean Sea Level (hourly tide observations); shown on topo sheets; accurate to 2 decimal places |
| Permanent B.M. | Survey of Pak / PWD | Established between GTS BMs; on fixed objects like kilometer stones, railway platforms, culverts, gate pillars |
| Arbitrary B.M. | Surveyor (assumed) | Elevation is just assumed — not tied to any fixed datum; used for small/local surveys only |
| Temporary B.M. | Surveyor (field) | Set at end of each day's work; work continues from here next day; set on km stones, tree roots, parapets; description must be written in level book |

# 5. STEPS IN LEVELLING

| # | Step | Formula |
| --- | --- | --- |
| 1 | Find the Height of Instrument (H.I.) by taking a Backsight on a Bench Mark | H.I. = R.L. of BM + B.S. reading |
| 2 | Find the R.L. of any other point by taking a staff reading (IS or FS) at that point | R.L. = H.I. − Staff Reading |

## Simple Levelling Example

Level is set up at O between points A and B. BS on A = 2.340 m, FS on B = 3.135 m. R.L. of A = 100 m

H.I. at O = 100 + 2.340 = 102.340 m Step 1: Find H.I.

R.L. of B = 102.340 − 3.135 = 99.205 m Step 2: Find R.L. of unknown point

Difference in elevation = 3.135 − 2.340 = 0.795 m B is lower than A by 0.795 m

# 6. TYPES OF LEVELLING

## Simple Levelling

- Used when BOTH points can be seen from ONE instrument position

- Just one setup — take BS on known point, FS on unknown point

## Differential Levelling (Compound / Continuous)

- Used when points are TOO FAR APART

- Used when DIFFERENCE IN ELEVATION is too large

- Used when OBSTACLES block the view between points

- Multiple instrument setups are needed — each connected by Change Points

Process for Differential Levelling: 1. Set up instrument — take BS on BM → calculate H.I. 2. Take IS on any intermediate points needed 3. Take FS on change point → record 4. Move instrument ahead — take BS on same change point → new H.I. 5. Repeat until final point reached

# 7. REDUCTION OF LEVELS (Booking Methods)

After taking readings in the field, we calculate the R.L. of each point using one of two methods:

## Method 1: Height of Collimation (H.I.) Method

### ▶ How it works:

- For each instrument setup, calculate the H.I.

- Then subtract each staff reading from H.I. to get R.L.

H.I. = R.L. of known point + B.S. reading First calculate H.I.

R.L. of any point = H.I. − Staff reading (IS or FS) Then find each R.L.

### ▶ Arithmetic Check:

ΣB.S. − ΣF.S. = Last R.L. − First R.L. Must be equal — if not, there's an error

LIMITATION: The arithmetic check does NOT verify intermediate sight (IS) calculations. Only BS, FS, and change points are checked.

### ▶ Worked Example — H.I. Method

| Station | B.S. | I.S. | F.S. | H.I. | R.L. | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.191 |  |  | 52.064 | 49.873 | T.B.M. 1 (BM) |
| 2 |  | 2.505 |  |  | 49.559 | A |
| 3 |  | 2.325 |  |  | 49.739 | B |
| 4 | 3.019 |  | 1.496 | 53.587 | 50.568 | C (C.P.) |
| 5 |  | 2.513 |  |  | 52.074 | D |
| 6 | 1.752 |  | 2.811 | 52.528 | 50.776 | E (C.P.) |
| 7 |  |  | 3.824 |  | 48.704 | T.B.M. 2 |
| CHECK: ΣBS = 6.962 ¦ ΣFS = 8.131 ¦ ΣBS − ΣFS = −1.169 ¦ Last RL − First RL = 48.704 − 49.873 = −1.169 ✓ |  |  |  |  |  |  |

## Method 2: Rise and Fall Method

### ▶ How it works:

- Compare CONSECUTIVE staff readings to each other

- If next reading is SMALLER → it's a RISE (ground went up)

- If next reading is LARGER → it's a FALL (ground went down)

- Add rise to previous R.L. or subtract fall to get new R.L.

| RISE Current reading < Previous reading Rise = Previous − Current R.L. = Previous R.L. + Rise | FALL Current reading > Previous reading Fall = Current − Previous R.L. = Previous R.L. − Fall |
| --- | --- |

### ▶ Arithmetic Check (3 checks — stronger than H.I. method):

ΣBS − ΣFS = ΣRise − ΣFall = Last R.L. − First R.L. ALL THREE must be equal

### ▶ Worked Example — Rise & Fall Method

| Stn | B.S. | I.S. | F.S. | Rise | Fall | R.L. | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.191 |  |  |  |  | 49.873 | T.B.M. 1 |
| 2 |  | 2.505 |  |  | 0.314 | 49.559 | A |
| 3 |  | 2.325 |  | 0.180 |  | 49.739 | B |
| 4 | 3.019 |  | 1.496 | 0.829 |  | 50.568 | C (C.P.) |
| 5 |  | 2.513 |  | 0.506 |  | 51.074 | D |
| 6 | 1.752 |  | 2.811 |  | 0.298 | 50.776 | E (C.P.) |
| 7 |  |  | 3.824 |  | 2.072 | 48.704 | T.B.M. 2 |
| CHECK: 6.962−8.131=−1.169 ¦ 1.515−2.684=−1.169 ¦ 48.704−49.873=−1.169 ✓ (All three equal!) |  |  |  |  |  |  |  |

## Which Method to Use?

| H.I. (Collimation) Method | Rise & Fall Method |
| --- | --- |
| ✅ Faster when many IS readings ✅ Good for mapping / setting out ❌ Only checks BS, FS, CPs (not IS) ❌ Risk of missing IS errors | ✅ 3 arithmetic checks (stronger) ✅ Best for establishing new TBMs ✅ Checks ALL points including IS ❌ Slower when many IS readings |

# 8. ADJUSTMENT OF LEVELS (Misclosure)

In real levelling, we start at a known BM and end at another known BM. Any difference between measured and actual final elevation is called MISCLOSURE.

Misclosure = Measured final R.L. − Known final R.L. A perfect survey has misclosure = 0

Allowable Misclosure = ± m√n m = constant (usually 5mm for construction); n = number of instrument positions

If misclosure is within the allowable limit → survey is acceptable → apply corrections proportionally.

- Correction sign = opposite of misclosure sign

- If misclosure is −0.006 m → correction = +0.006 m → distribute evenly across all points

# 9. SOURCES OF ERRORS IN LEVELLING

| Instrumental Errors | Natural Errors | Personal Errors |
| --- | --- | --- |
| • Line of sight not horizontal • Cross hair not horizontal • Staff incorrect length • Tripod legs loose | • Curvature of Earth • Atmospheric Refraction • Temperature variations • Wind (instrument vibration) • Settlement of instrument/CP | • Bubble not centred • Parallax error • Incorrect staff reading • Improper staff handling • Staff not held vertical |

# 10. QUICK REVISION — ALL FORMULAS

| What to Find | Formula |
| --- | --- |
| Height of Instrument (H.I.) | H.I. = R.L. of known point + B.S. |
| R.L. of a point (H.I. method) | R.L. = H.I. − Staff reading |
| Rise | Rise = Previous reading − Current reading (when current is smaller) |
| Fall | Fall = Current reading − Previous reading (when current is larger) |
| R.L. when Rise | R.L. = Previous R.L. + Rise |
| R.L. when Fall | R.L. = Previous R.L. − Fall |
| Arithmetic Check (H.I.) | ΣBS − ΣFS = Last R.L. − First R.L. |
| Arithmetic Check (R&F) | ΣBS − ΣFS = ΣRise − ΣFall = Last R.L. − First R.L. |
| Gradient | Gradient = Horizontal Distance ÷ Difference in Elevation |
| Allowable Misclosure | ± m√n (m=5mm typically, n=no. of instrument positions) |

Good Luck on Your Exam!